## GEO-ELECTRIC INVESTIGATION OF UNDERGROUND LEACHATE DISTRIBUTION AT A CLOSED LANDFILL IN SOUTHWESTERN ONTARIO, CANADA

JOSHI, Siddharth Dilip1, YANG, Jianwen1, SERERES, Clayton2, and TAMR, Radwan3, (1) Earth and Environmental Sciences, University of Windsor, 401 Sunset Avenue - University of Windsor, Windsor, ON N9B 3P4, Canada, joshis@uwindsor.ca, (2) Project Manager, Environment, Genivar Inc, 4510 Rhodes Driver, Suite 720, Windsor, ON N8W 5K5, Canada, (3) Senior Project Engineer, Genivar Inc, 4510 Rhodes Driver, Suite 720, Windsor, ON N8W 5K5, Canada

Contamination due to landfill leachate can cause disastrous effects to aquifers used for groundwater supply. In this study, two geo-electric techniques were employed to investigate the underground leachate distribution and the subsurface geology at a closed site in southwestern Ontario, Canada. Conductivity mapping was first conducted over the study site using a frequency-domain EM terrain conductivity meter, which reveals an anomalous zone with the apparent conductivity values ranging between 130-500mS/m, concentrated in the 200m (S-N) by 80m (W-E) rectangle area at western half of the site. The DC resistivity survey was then carried out in the same area with 8 S-N profiles and 3 W-E profiles measuring 200m in length using a Wenner-α configuration. This arrangement of electrodes has an approximate exploration depth of 30m, which aids in outlining the waste and upper sand aquifer, the underlying silt/sand aquitard, and the lower sand aquifer. Our resistivity survey results exhibit an inconsistency in the clay cap thickness on almost all the profiles conducted. The results also indicate that the contamination plume is mainly travelling southwards at the base of the upper aquifer just below the waste, with a minor vertical component into the upper weathered portion of the silt/sand aquitard at some locations. No contamination plumes seem to exist in the lower sand aquifer. These findings are critical in assessing the current leachate conditions and in evaluating the existing compliance monitoring plan for potential implementation at the site or other sites in elsewhere.

Abstract ID#: 218499
Password: 885821
Meeting: North-Central Section - 47th Annual Meeting (2-3 May 2013)
Session Type: Topical/Theme
Selection: T2. Applications of Near-Surface Geophysics
Title: GEO-ELECTRIC INVESTIGATION OF UNDERGROUND LEACHATE DISTRIBUTION AT A CLOSED LANDFILL IN SOUTHWESTERN ONTARIO, CANADA
Key Words: Geo-electric, Leachate, Southwestern Ontario, EM conductivity, DC resistivity
Presentation Format: Poster
Discipline Categories: Geophysics/Tectonophysics/Seismology

Scheduled For: 

Abstract Submission Fee: Paid (gsa-2013NC-6548-5075-2077-0372)

### First Author

Student Presenting
Siddharth Dilip Joshi
(Graduate student)
Earth and Environmental Sciences, University of Windsor
401 Sunset Avenue - University of Windsor
Windsor, ON N9B 3P4
Canada 
Office Phone: 519 253 3000 ext: 2181 
Email: joshis@uwindsor.ca 
Confirmation Email: joshis@uwindsor.ca

### Second Author

Jianwen Yang
Earth and Environmental Sciences, University of Windsor
401 Sunset Avenue - University of Windsor
Windsor, ON N9B 3P4
Canada 
Office Phone: 519 253 3000 ext: 2181 
Email: jianweny@uwindsor.ca

### Third Author

Clayton Sereres
Project Manager, Environment, Genivar Inc
4510 Rhodes Driver, Suite 720
Windsor, ON N8W 5K5
Canada 
Office Phone: 519 974 5887 ext. 221 
Email: clayton.sereres@genivar.com

### Fourth Author

Radwan Tamr
Senior Project Engineer, Genivar Inc
4510 Rhodes Driver, Suite 720
Windsor, ON N8W 5K5
Canada 
Office Phone: 519 974 5887 ext. 224 
Email: radwan.tamr@genivar.com